#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <SoftwareSerial.h>

#include <TinyGPS++.h>

// ---------------- PIN DEFINITIONS ----------------

#define CANCEL_BUTTON 2

#define BUZZER 8

#define LED 9

// GPS: Arduino RX, TX

SoftwareSerial gpsSerial(4, 3);

// GSM: Arduino RX, TX

SoftwareSerial gsmSerial(7, 6);

TinyGPSPlus gps;

LiquidCrystal_I2C lcd(0x27, 16, 2);

// ---------------- PHONE NUMBERS ----------------

// Replace these with actual emergency/contact numbers

const char hospitalNumber[] = "+91XXXXXXXXXX";

const char policeNumber[]   = "+91XXXXXXXXXX";

// ---------------- ACCIDENT SETTINGS ----------------

const float ACCIDENT_THRESHOLD = 2.5;  // approximately 2.5g

const unsigned long CANCEL_TIME = 10000; // 10 seconds

// MPU6050 I2C address

const int MPU_ADDR = 0x68;

bool accidentDetected = false;

// ----------------------------------------------------

// MPU6050 INITIALIZATION

// ----------------------------------------------------

void initMPU6050()

{

Wire.beginTransmission(MPU_ADDR);

Wire.write(0x6B);       // Power management register

Wire.write(0x00);       // Wake up MPU6050

Wire.endTransmission(true);

// Accelerometer ±8g

Wire.beginTransmission(MPU_ADDR);

Wire.write(0x1C);

Wire.write(0x10);

Wire.endTransmission(true);

}

// ----------------------------------------------------

// READ ACCELERATION FROM MPU6050

// ----------------------------------------------------

float readAcceleration()

{

int16_t ax, ay, az;

Wire.beginTransmission(MPU_ADDR);

Wire.write(0x3B);

Wire.endTransmission(false);

Wire.requestFrom(MPU_ADDR, 6, true);

ax = Wire.read() << 8 | Wire.read();

ay = Wire.read() << 8 | Wire.read();

az = Wire.read() << 8 | Wire.read();

// ±8g range = 4096 LSB/g

float Ax = ax / 4096.0;

float Ay = ay / 4096.0;

float Az = az / 4096.0;

float totalAcceleration =

sqrt((Ax * Ax) + (Ay * Ay) + (Az * Az));

return totalAcceleration;

}

// ----------------------------------------------------

// SEND AT COMMAND TO GSM

// ----------------------------------------------------

void sendGSMCommand(String command, int waitTime)

{

gsmSerial.println(command);

delay(waitTime);

}

// ----------------------------------------------------

// SEND SMS

// ----------------------------------------------------

void sendSMS(const char *number, String message)

{

gsmSerial.println("AT");

delay(1000);

gsmSerial.println("AT+CMGF=1");

delay(1000);

gsmSerial.print("AT+CMGS=\"");

gsmSerial.print(number);

gsmSerial.println("\"");

delay(1000);

gsmSerial.print(message);

delay(500);

gsmSerial.write(26);   // CTRL+Z

delay(5000);

}

// ----------------------------------------------------

// GET GPS LOCATION

// ----------------------------------------------------

bool getGPSLocation()

{

unsigned long startTime = millis();

while (millis() - startTime < 15000)

{

while (gpsSerial.available())

{

gps.encode(gpsSerial.read());

}

if (gps.location.isValid())

{

return true;

}

}

return false;

}

// ----------------------------------------------------

// CANCEL ALERT

// ----------------------------------------------------

bool checkCancel()

{

unsigned long startTime = millis();

while (millis() - startTime < CANCEL_TIME)

{

while (gpsSerial.available())

gps.encode(gpsSerial.read());

lcd.setCursor(0, 0);

lcd.print("ACCIDENT ALERT  ");

lcd.setCursor(0, 1);

lcd.print("Cancel: ");

lcd.print((CANCEL_TIME - (millis() - startTime)) / 1000);

lcd.print(" sec ");

digitalWrite(BUZZER, HIGH);

digitalWrite(LED, HIGH);

if (digitalRead(CANCEL_BUTTON) == LOW)

{

digitalWrite(BUZZER, LOW);

digitalWrite(LED, LOW);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("ALERT CANCELLED");

delay(2000);

return true;

}

delay(100);

}

return false;

}

// ----------------------------------------------------

// SEND EMERGENCY ALERT

// ----------------------------------------------------

void sendEmergencyAlert()

{

digitalWrite(BUZZER, HIGH);

digitalWrite(LED, HIGH);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("GETTING GPS...");

lcd.setCursor(0, 1);

lcd.print("PLEASE WAIT");

bool locationAvailable = getGPSLocation();

String message;

if (locationAvailable)

{

double latitude = gps.location.lat();

double longitude = gps.location.lng();

message = "ACCIDENT ALERT!\n";

message += "Possible accident detected.\n";

message += "Location:\n";

message += "https://maps.google.com/?q=";

message += String(latitude, 6);

message += ",";

message += String(longitude, 6);

}

else

{

message = "ACCIDENT ALERT!\n";

message += "Possible accident detected.\n";

message += "GPS location unavailable.";

}

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("SENDING ALERT...");

// GSM serial must be selected

gpsSerial.listen();

delay(100);

gsmSerial.listen();

sendSMS(hospitalNumber, message);

sendSMS(policeNumber, message);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("ALERT SENT");

lcd.setCursor(0, 1);

lcd.print("HELP REQUESTED");

delay(5000);

digitalWrite(BUZZER, LOW);

digitalWrite(LED, LOW);

}

// ----------------------------------------------------

// SETUP

// ----------------------------------------------------

void setup()

{

pinMode(CANCEL_BUTTON, INPUT_PULLUP);

pinMode(BUZZER, OUTPUT);

pinMode(LED, OUTPUT);

digitalWrite(BUZZER, LOW);

digitalWrite(LED, LOW);

Serial.begin(9600);

gpsSerial.begin(9600);

gsmSerial.begin(9600);

Wire.begin();

lcd.init();

lcd.backlight();

initMPU6050();

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("ACCIDENT ALERT");

lcd.setCursor(0, 1);

lcd.print("SYSTEM READY");

delay(3000);

lcd.clear();

}

// ----------------------------------------------------

// MAIN LOOP

// ----------------------------------------------------

void loop()

{

// Continuously read GPS data

gpsSerial.listen();

while (gpsSerial.available())

{

gps.encode(gpsSerial.read());

}

float acceleration = readAcceleration();

Serial.print("Acceleration: ");

Serial.print(acceleration);

Serial.println(" g");

lcd.setCursor(0, 0);

lcd.print("SYSTEM NORMAL   ");

lcd.setCursor(0, 1);

lcd.print("ACC:");

lcd.print(acceleration, 2);

lcd.print("g       ");

// Check accident condition

if (acceleration >= ACCIDENT_THRESHOLD)

{

accidentDetected = true;

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("ACCIDENT");

lcd.setCursor(0, 1);

lcd.print("DETECTED!");

delay(1000);

// Give user 10 seconds to cancel

bool cancelled = checkCancel();

if (!cancelled)

{

sendEmergencyAlert();

}

accidentDetected = false;

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("SYSTEM READY");

delay(3000);

}

delay(100);

}